Model for speed from throttle

For the formula, speed is in m/s. Time is in seconds, sampled every 100 ms.

Computations are done using mm/s, which enables integer arithmetic. The Arduino has no floating-point processor and integer arithmetic is considerably faster.

Speed(t) is present speed.

Speed(t-1) is previous speed

The throttle voltage ranges from 0 to 5V, but voltages below about 1 V don’t do anything and voltages over 4V or so have no additional effect. The data was digitized to the 0 to 255 range.  This is a bit different than the Arduino Analog In pin, which converts the voltage to a digital value from 0 to 1023. You may need a suitable multiplier.

For these tests, throttle is a unitless number between 50 and 227.

Mean_throttle(t) = 18i=t-10t-3throttle(i)

Note that the summation produces a delay of 0.75 s from throttle to speed.

Estimated_speed(t) = max(0, 13.6 * (Mean_throttle(t) – 65)/(227-65) )

Momentum (t) = 0.9296 * speed(t-1)

Speed(t) = max (Momentum(t), Estimated_speed(t))

Good coding could be

#define FRICTION 0.9296

#define MIN_EFFECTIVE_THROTTLE 65

#define MAX_EFFECTIVE_THROTTLE 227