Simulator

The bridge board connects an Arduino Due (DBW) to a second Due (Router). The original intent was for the router to connect to a simulator on a PC. The first priority of the simulator is much simpler, and can be implemented on the Due without needing to connect to a PC.

The simulator bridge was the result of two capstone projects, described in Final_Report_Spring_2020.pdf. This version used an Arduino Mega and two Dues. We then attempted to make it into a commercial product based on three Dues. It is documented on  https://github.com/elcano/Bridge

The ultimate goal is to be able to develop software without depending on the physical trike. Interface to the CARLA simulator was difficult. We should be able to get acceptable results from a simpler simulation. It would be good if we could come up with an automated regression suite that checked any new code against the simulator to make sure it does not introduce new problems.

## Stage 1

Inputs to Router from DBW:

| DBW pin | Signal | Meaning | Router pin |
| --- | --- | --- | --- |
| DAC0 | THROTTLE | >= 205 moves vehicle up to 1000 max | A0 |
| D40 | BRAKE_VOLT | Hi=Apply brakes; Lo=Hold brakes | D42 |
| D44 | BRAKE_ON | Hi=Brakes on; lo=Brakes off | D48 |
| D26 | L_TURN | L hi, R lo = turn farther left; both lo = no change | D4 |
| D28 | R_TURN | L lo, R hi = turn farther right; both hi = no change | D2 |

Outputs from Router to DBW:

| DBW pin | Signal | Meaning | Router pin |
| --- | --- | --- | --- |
| D47 | IRPT_WHEEL | One pulse per wheel revolution | D47 |
| A10 | L_SENSE | Wheel angle sensor on left steering column | DAC0 |
| A11 | R_SENSE | Wheel angle sensor on right steering column | DAC1 |
| D46 |  | Pulse throttle from RC | D51 |
| D48 |  | Pulse steer from RC | D52 |

Be sure that JP1 and JP3 have jumpers between pins 2 and 3 to enable L_SENSE and R_SENSE.

See Settings.h for wheel diameter and values for wheel angle sensor.

Write a program for the Due that will produce appropriate values for the sensors from the inputs. Connect the Router Arduino to a serial monitor and keep track of the vehicle position (meters with +Y=north, +X=east), orientation (compass bearing in degrees) and speed (m/s or km/hr). This can be captured as a CSV file and displayed graphically in a spreadsheet. For this effort, the DBW Arduino will be programmed to run on a fixed course with no user input. The CSV file can be captured on a PC from serial output. It could also be written to the SD card on the bridge board with the router SPI. Using the router SD card requires 3 jumpers in parallel on JP2. You may need to connect a wire from Sensor Hub Pin 35 (SD card chip select) to TP12 (router pin D20) and use D20 as chip select.

We captured data in 2019 for the vehicle on a test stand with no load. The model for throttle to speed is given in “Model for speed from throttle.docx”. This is a good place to start. Data from 2019 is on https://github.com/elcano/elcano/tree/master/Simulator In the Data folder, files Throttle_Data2.xlxs and Throttle_Notes9_3.docx are of interest.  This spreadsheet graphs the model to the data.

## Stage 2

The second iteration of the simulator would be an arial 2D view of the vehicle’s position. Use the PC keyboard or mouse to set the control commands from the RC unit. The pulse for throttle is 1.5 ms wide for no motion, 1 ms for brake and 1.6 to 2.0 ms for throttle. The steer pulse is 1.5 ms for straight ahead, less for left turn and more for right turn [check DBW code to make sure I have the right directions].

## Stages 3 and 4

The third stage of the simulator is to map (X,Y) position to a latitude and longitude. The GPS sentence is output on router pin D15 (Rx3). I’ll supply details on GPS when you get to this point. The GPS signal will go to the Jetson Nano.

None of this requires fancy graphics. The final stage of the simulator will be to add the graphics, using Unity, CARLA, or another system. The graphics will enable the camera on the Jetson Nano to point at the screen to detect obstacles and stay on the road.